Smart Pool Table Unit — Phase 1

Standalone GSM-Based Payment & Auto-Release Unit — Technical Specification

# 1. Design Intent

Phase 1 is a fully self-contained unit. It requires no internet connection, no cloud server, and no venue WiFi. A GSM module with a SIM card receives the M-Pesa payment confirmation SMS directly, the on-board controller parses it, converts the amount into game credits at a configurable price, and displays the queue on an LCD. The customer presses a single button to release the balls, and one credit is deducted.

This gives the business a production-grade, deployable unit at minimum complexity, while keeping a clean upgrade path to the networked multi-table system described in the main proposal.

# 2. Operating Principle

- The SIM inside the device is the number registered to receive the Till's M-Pesa confirmation messages.

- On payment, M-Pesa sends a confirmation SMS; the GSM module receives it and passes it to the controller over UART.

- The controller validates the sender and message format, extracts the amount and transaction code, and credits games = amount ÷ price per game.

- Any remainder is retained as a balance toward the next game.

- The LCD continuously displays the number of games queued and the current price.

- Each button press fires the solenoid for a set pulse duration, releasing the balls, and decrements the credit count by one.

# 3. Power Architecture

Mains power does not feed the electronics directly. The 12V adapter passes through the master ON/OFF switch into a charger/BMS module, which maintains the 18650 battery pack. The battery rail then supplies all loads at all times. This is a true pass-through UPS topology: there is no switchover delay when mains fails, and the battery naturally absorbs the solenoid's inrush current, which would otherwise cause brown-out resets of the controller.

# 4. Bill of Materials

| Component | Function | Notes |
| --- | --- | --- |
| ESP32 | Main controller: SMS parsing, credit accounting, LCD, button, relay control | Adequate GPIO and on-board flash for non-volatile credit storage |
| GSM module + SIM holder | Receives M-Pesa confirmation SMS over the cellular network | SIM7600 preferred for wider voltage tolerance; SIM800L is a lower-cost alternative |
| 16x2 I2C LCD | Customer-facing display: games queued, price, prompts, status messages | I2C keeps wiring to two signal lines |
| Tactile push button | Customer trigger to release balls and consume one credit | Debounced in firmware |
| Relay module (5V) | Switches the solenoid circuit from the controller's logic pins | Isolates the actuator's current draw from control electronics |
| 12V solenoid | Trips the table's existing ball-release lever | Reuses the original coin mechanism; no redesign of the ball lock |
| Buck converter (12V→5V) | Supplies the controller and LCD | Shared battery rail source |
| GSM regulator (→4.0V) + 1000µF capacitor | Dedicated supply for the GSM module | Mandatory — see Section 5 |
| 18650 cells (2–3) + BMS/charger | Battery backup and power conditioning | Charges from mains; carries the load continuously |
| 12V mains adapter | Primary power input and battery charging source | Feeds the charger, not the board |
| Master ON/OFF switch | Single physical control for the entire unit | Positioned immediately after the power inlet |

# 5. Critical Design Risks

## 5.1 GSM power supply

The GSM module draws current spikes of up to approximately 2A during network transmission and operates below 5V. If it shares a rail with the controller, these spikes cause resets and dropped messages. It must be given its own regulated supply with a large bulk capacitor placed directly at its power pins. This is the most common cause of failure in builds of this type and is treated as a mandatory design requirement, not an optimisation.

## 5.2 Credit persistence across power loss

Game credits represent money already paid. If the unit resets while a customer holds unused credits, those credits must survive. The credit count and balance are therefore written to the controller's non-volatile flash on every change, not held in volatile memory.

## 5.3 Duplicate transaction handling

If the GSM module re-reads its message inbox after a reboot, previously processed payments could be credited twice. The firmware maintains a rolling log of recent transaction codes in flash and ignores any message whose code has already been processed.

## 5.4 Single-till limitation

Because the unit relies on receiving the Till's confirmation SMS, one SIM corresponds to one Till. This design is therefore inherently single-table. Serving multiple tables on a shared Till requires the networked architecture described in the main proposal. This should be understood as a deliberate Phase 1 scope boundary.

# 6. Firmware Logic

CREDITS and BALANCE held in non-volatile flash

PRICE_PER_GAME configurable (default 100)

on_sms_received(msg):

if sender is MPESA and message format valid:

code, amount = parse(msg)

if code in recent_transaction_log: ignore

else:

BALANCE += amount

games = BALANCE / PRICE_PER_GAME

CREDITS += games; BALANCE = remainder

save_to_flash(); display("Paid! N games")

on_button_press():

if CREDITS > 0 and battery above threshold:

pulse_relay(400ms); CREDITS -= 1

save_to_flash(); display("Enjoy! N left")

else:

display("No games. Pay Till XXXXX")

# 7. Handled Conditions

| Condition | System behaviour |
| --- | --- |
| Partial payment below one game price | Amount retained as a balance and displayed; credited once the total reaches the game price |
| Payment confirmation delayed by network | Display shows a waiting message so the customer does not pay a second time |
| Mechanism jams after release | Concealed staff override button grants a release without consuming a credit |
| Battery below safe threshold | Warning displayed; solenoid actuation refused to avoid a partial throw jamming the mechanism |
| Mains power failure | Battery carries the load with no interruption; credits and balance unaffected |
| Price change required | Adjustable without reflashing, via an administrative SMS from a whitelisted number |

# 8. Remote Administration

Because the unit already has a cellular connection, the owner can administer any deployed unit from any location by SMS from a whitelisted number, with no additional hardware:

- Set the price per game

- Request a status report (credits outstanding, battery level, signal strength)

- Grant a complimentary game remotely

# 9. Upgrade Path

The firmware is structured so that payment detection is a replaceable module. The enclosure, display, button, actuator and power design remain unchanged across all phases:

- Phase 1: SMS-based detection, local credit accounting, no internet dependency

- Phase 2: same hardware, payment detection switched to an online payment API over the existing cellular data connection, enabling per-table identification and digital receipts

- Phase 3: addition of a central hub and per-table nodes, plus the management application for owners and location managers

# 10. Approval Points

- Confirm the default price per game and whether location managers may adjust it

- Confirm the GSM module selection, balancing unit cost against supply robustness

- Confirm whether a concealed staff override button is included in the pilot build

- Approve construction of a single pilot unit for mechanical testing against an existing table